# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: „Supply of module system for reaction at crisis situations in Petrich under project CB006.2.12.132“	p 1 /14

Publication reference: CB006.2.12.132 – LP – Supply 1

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1. | MODULE SYSTEM FOR REACTION AT CRISIS SITUATIONS The module system must be based on a pickup truck and should consist of two separate modules according to their purpose, which should be quickly and easily replaced each other according to the response needs: flood and forest fire. The system should include the following components with the following minimum specifications required: |  |  |  |
| 1.1. | Chassis (with additional equipment) - 1 pc: - Pick-up vehicle; - Movement: 4х4 with differential locker; - Payload: at least 1 000 kg; - Cab: 1+4 with 4 doors; - Dimension of cargo compartment (L/W/H) at least 1 510/1 460/390 mm; - Hardtop from aluminium corrugated sheet with height at least to the height of the cabin and one rear door. The hardtop should be able to be mounted and removed quickly and easily by 2 crew members; - Sliding bed for assembling of the different modules - Rear camera; - Tyres: at least 17“ off-road; - Front angle: ≥ 29о; - Rear angle: ≥ 22о; - Angle of slope: ≥ 24о; - Side tilt angle: ≥ 45о; - Ground clearance: at least 200 mm; - Engine: diesel, 4 cylinders with capacity at least 2 400 cm3, EURO 6; - Power: at least 130 kW; - Average fuel consumption: up to 7,0 l/100 km; |  |  |  |
|  | - СО2: up to 170 g/km; - Gear box: manual, at least 5+1 gears; - Rear suspension: leaf springs; - ABS, ESP - Front and rear fog lights; - 1 set orange signal light-bar mounted on the roof of the cabin, electronic siren with power of at least 100 W; - 1 pc. Digital mobile radio 136-174 MHz, 25 W, set with antenna, inside speaker, vehicle stand and microphone; - 2 pcs. Digital portable radio, 136-174 MHz, 5 W. IP 54 or IP 55. Set with Li-Ion battery pack at least 1 9000 mAh, antenna, charger 230 V 50 Hz, charger 12 V for vehicle; - Programing software and interface cable/s |  |  |  |
| 1.2. | Module “Flood” (1 set) - module for flood response. This module must be equipped with the following equipment: |  |  |  |
| 1.2.1. | Power generator – 1 pc: - 4-stoke gasoline engine - Manual start - Nominal power of the generator: at least 5 kW - Automatic voltage regulator (AVR) - 230 V 50 Hz - At least two sockets 230 V 16 A - Weight: up to 95 kg |  |  |  |
| 1.2.2. | Mobile light mast – 1 pc: - Portable; - Three section masts with a working height of not less than 3 500 mm; - Lighting devices 2 х min. 100 W 230 V; - Gasoline power generator at least 1 kVA, silent type; - Stabilizers; - min. 10 m power cable. |  |  |  |
| 1.2.3. | Submersible water pump – 1 pc: - Vertical, one-stage, made from stainless steel; - Cable length ≥ 5 m - Start / stop float - Fluid temperature from 0 ° C to + 50 ° C - Flow rate: ≥ 8 m3 / h - Working pressure: ≥ 5 m - Operating voltage 230 V 50 Hz - Engine power: ≥ 300 W - Solid particle size ≥ 10 mm - Safety rope ≥ 10 m - Weight ≤ 6,6 kg - Outlet Storz C |  |  |  |
| 1.2.4. | Floating water pump – 1 pc: - Max. flow 750 l/min - Water throw at least 30 m - Outlet Storz C - Power at least 5 kW - Weight up to 25 kg - Security rope min. 20 m |  |  |  |
| 1.2.5. | Trash pump 2” – 1 pc: - Flow at least 700 l/min - Water throw at least 26 m - Outlet Storz C - Power at least 3.5 kW - Weight up to 65 kg - Particles ≥ 24 mm |  |  |  |
| 1.2.6. | Hoses – 5 pcs: - Length 20 m - Diameter Ø 52 mm - Couplings Storz C - min. PN 16 bar - Acc.to DIN 14 811 or equivalent |  |  |  |
| 1.2.7. | Overalls for water rescue – 2 pcs: It should consist of rubber boots and trousers. The trousers should be made of a one-sided rubberized waterproof fabric. The seams should be sealed. |  |  |  |
| 1.2.8. | Inflatable boat – 1 pc: - For min. 4-6 persons - Length ≥ 2 m - Width ≥ 1.1 m - Cameras ≥ 2 - Security rope - Paddles - Air pump |  |  |  |
| 1.2.9. | Floating rope – 2 pcs: - Material: PP or equivalent - Colour: orange or equivalent bright colour - Diameter Ø 8 mm + 5 % - Length min. 50 m |  |  |  |
| 1.2.10. | Life jacket – 4 pcs: - automatic, inflatable - buoyancy ≥ 165 N - Red colour or equivalent bright colour |  |  |  |
| 1.2.11. | Suction hoses – 1 set: - Length ≥ 5,5 m - couplings Storz C |  |  |  |
| 1.2.12. | Suction strainer – 1 pc: - couplings Storz C |  |  |  |
| 1.2.13. | Ceiling hook – 1 pc: - Wooden handle Ø 35 – 40 mm, two part - Two couplings - Length min. 2,5 m - Weight up to 3,8 kg |  |  |  |
| 1.2.14. | Fuel canister – 1 pc: - Metal petrol canister - UN-certified - min. 20 l volume - Rigid spout - Weight up to 4,5 kg (with spout) |  |  |  |
| 1.2.15. | Spry colours – 1 set: - in metal cans - at least 400 ml - colours: red, yellow and green |  |  |  |
| 1.2.16. | Head lamp – 4 pcs: - maximum power at least 350 lm - range at least 90 m - accu battery with capacity at least 1200 mAh - weight up to 95g - min. IP X4 |  |  |  |
| 1.2.17. | Rope ladder – 1 pc: - made from synthetic rope - working range at least 10 m - load capacity not less than 150 kg |  |  |  |
| 1.3. | Module “Forest fire” - module for forest fire response. This module must be equipped with the following equipment: |  |  |  |
| 1.3.1. | Ultra-high pressure extinguishing system – 1 set: - Working pressure at least 100 bar; - 1 pc. reel with rubber hose with metal internal braid (core) with a length of at least 100 m. With handle winding and locking device; - Engine: 4-stroke gasoline engine with electric start. Power not less than 10 kW (14 hp). Air cooling system; - Pump assembly for high pressure with a maximum flow rate of not less than 42 l / min at a maximum pressure of no less than 100 bar; - L-shaped reservoir from GRP, with a capacity of not less than 400 l, including not less than 380 l of water and not less than 20 l foam agent; - Foam supply system, for foam supply with low and medium multiplicity. - Pistol-type nozzle for water and foam, with the possibility of submission of at least two types of water jets and foam jet. The nozzle should be designed to create maximum working pressure and ensure maximum flow of the installation. - Range of the jets: not less than 18 m in compact jet, not less than 12 m in spray jet, and not less than 10 m in foam jet. - Suction device with no less than 6 m suction pipe with Storz type connector and protective cap. |  |  |  |
| 1.3.2. | Portable water pump – 1 pc: - gasoline engine - inlet: 1" - outlet: 1" Storz D - water throw: ≥35 m - flow: ≥ 120 l/min - Dept: ≥ 6 m - Suction hose ≥ 6 m - Suction strainer - Security rope ≥ 10 m |  |  |  |
| 1.3.3. | Hoses – 5 pcs: - Length min. 20 m - Diameter Ø 25 mm - Couplings Storz D - min. PN 16 bar - Acc.to DIN 14 811 or equivalent |  |  |  |
| 1.3.4. | Nozzle – 1 pc: - Flow ≥ 100 l/min - Coupling Storz D |  |  |  |
| 1.3.5. | Floodlights – 4 pcs: - With 2 pcs. LED sources with a total luminous flux of at least 200 lm. - Tilting on at least three fixed positions lighting head (at 0°, 45° and 90°). - Light digital indication of the time remaining for lighting in hours and minutes. - With at least three modes of light for operation with a Charging not less than 4 h at full intensity and not less than 8 h at minimal intensity. - Protection against water and dust not less than IP 67. - Weight not more than 500 g. - Accessories: Spare glass. Charger 230 V. - In the car to be installed charger for charging at least 1 pc. flashlight from electrical installation of the car. |  |  |  |
| 1.3.6. | Fire battledore – 2 pcs: - With a wooden handle. - Type "metal broom" or equivalent |  |  |  |
| 1.3.7. | Water backpack – 2 pcs: - Water tank ≥ 18 l - Nozzle with compact and fog jets - Range of the compact jet ≥ 10 m |  |  |  |
| 1.3.8. | Sanitary device – 1 set: - Apparatus for automatic defibrillation and CPR - Colour Screen ≥ 10cm - ECG and CPR curve display - Built-in battery ≥ 5 years life - Weight with battery ≤ 2kg - Voice recording during resuscitation - Operating temperature min. range -10 ° C to + 55 ° C - ≥ IP 66 protection against water and dust - Menu and voice commands in Bulgarian -Should include the following accessories: Defibrillation electrodes - 1pc, CPR sensor - 1pc. |  |  |  |
| 1.3.9. | Motor chainsaw – 1 pc: - Gasoline engine - Power: at least 2.5 kW - Chain .325” - Bar 18” - Weight: up to 5,5 kg |  |  |  |
| 1.3.10. | Protective helmet – 4 pcs: - Acc.to EN 12492: 2012 or equivalent standard - Ventilated, with adjustable size of №52 up to №66 |  |  |  |
| 1.3.11. | Protective suit - 4 sets: - Acc. to new standard EN ISO 13688:2013 (former standard EN 340:2003) or equivalent standard, new standard ISO 11612:2015 (former standard ISO 11612:2008) or equivalent standard, EN 15614:2007 or equivalent standard and new standard EN 1149-5:2019 (former standard EN 1149-5:2008) or equivalent standard - Colour of the set – to be offered by the supplier - Reflective strips. |  |  |  |
| 1.3.12. | Protective gloves – 4 pairs: - Acc.to EN 659 (extended EN 659:2003+A1:2008) or equivalent standard |  |  |  |
| 1.3.13. | Protective boots – 4 pairs: - Acc.to EN 15090:2012 or equivalent standard - type F2A, according to EN 15090:2012 or equivalent standard - The weight of a pair of boots to be in accordance to the standard EN 15090:2012 or equivalent standard. |  |  |  |